MINISTRY REGULATIONS 

Excerpts from

Occupational Health and Safety Act (OHSA)

and

Regulations for Industrial Establishments
R.R.O. 1990, Reg. 851, as amended by .R.O. 1990, Reg. 834

 “1. (1) 

In this Act

“competent person” means a person who, 

is qualified because of knowledge, training and experience to organize the 

work and its performance,

is familiar with this Act and the regulations that apply to the work, and

c)  as knowledge of any potential or actual danger to health or safety in the 

     workplace;…” 



page 1
“1.
In this regulation,

“lifting device” means a device that is used to raise or lower any material or object and includes its rails and other supports but does not include a device to which the Elevating Devices Act applies;…”. 



Definitions page R-2
Regulation 851, s. 51   


“(1) A lifting device shall, 

be so constructed, of such strength and be equipped with suitable ropes, chains, 

slings and other fittings so as to adequately ensure the safety of all workers;

be thoroughly examined by a competent person to determine its capability of 

handling the maximum load as rated,

prior to being used for the first time, and

thereafter as often as necessary but not less frequently than 

recommended by the manufacturer and in any case, at least once a 

year, and                                                                                                                                      

a permanent record shall be kept, signed by the competent person 

doing the examination;

OHSA Regulations for Industrial Establishments





  

be plainly marked with sufficient information so as to enable the operator of the 

device to determine the maximum rated load that the device is capable of lifting 

under any operating condition;”

“(d) have a cab, screen, canopy guard or other adequate protection for the 

operator where the operator may be exposed to the hazard of falling material;

when it is a pneumatic or hydraulic hoist, have controls that automatically 

return to their neutral position when released.

(2) A lifting device shall be operated, 

only by, 

i) a competent person, or 

ii) a worker being instructed who is accompanied by a competent person; and

in such a way that,

(i)  no part of the load passes over any worker,

(ii) where a worker may be endangered by the rotation or uncontrolled motion 

      of a load, one or more guide ropes is used to prevent rotation or other 

      uncontrolled motion, and

(iii) subject to subsection (3), when its load is in a raised position the controls 

       are attended by an operator.

(3) Subclause (2) (b) (iii) does not apply to, 

a hydraulic hoist that supports the load from below and is fixed in one location; 

and

an assembly line hoist temporarily unattended during a stoppage of the assembly 

line.

(4) Hoisting controls operated from other than a cab or cage shall,

be located so that they can be operated at a safe distance from a load being lifted;          

and
(b) automatically return to their neutral position when released.”

OHSA Regulations for Industrial Establishments







(5) Where a lifting device is equipped with limit switches, the switches shall, 

(a) automatically cut off the power and apply the brake; and 
not be used as an operating control unless designed for such use, in which case a second limit switch shall be located behind the control limit switch.” 

Reference: Occupational Health and Safety Act and Regulations for Industrial Establishments, Jan 2002, pages R-28 to R-29

Regulation 851, s. 56

“Where the operator of a vehicle, mobile equipment, crane or similar material handling equipment does not have a full view of the intended path of travel or the vehicle, mobile equipment, crane or similar material handling equipment or its load, the vehicle, mobile equipment, crane or similar material handling equipment shall only be operated as directed by a signaller who is a competent person and who is stationed,

a) in full view of the operator;

b) with a full view of the intended path of travel of the vehicle, mobile equipment, crane or similar material handling equipment and its load; and

c) clear of the intended path of travel of the vehicle, mobile equipment, crane or similar material handling equipment and its load. R.R.O. 1990, Reg. 851, s.56”

Reference: Occupational Health and Safety Act and Regulations for Industrial Establishments, Jan 2002, pages R-31 to R-32

LDL CRANE TRAINING

PAGE  
3
LDL Crane Training


